Hpdcache Uvm Verification

Summary

Thesis UVM verification testbench for cv-hpdcache L1 Data Cache with Domino Prefetcher

Details

- UVM verification testbench for a cv-hpdcache L1 Data Cache integrated with a custom Domino Prefetcher

- Targets the cv32a6_imac_sv32 configuration

- Environment: QuestaSim 23.3 Starter Edition with UVM 1.1d

- Project lives at D:/HCMUS/THESIS/cv-hpdcache-master; UVM testbench at D:/HCMUS/UVM/

- DUT is hpdcache_wrapper.sv

- Key DUT parameters: PA=56-bit, SETS=64, WAYS=8, REQ_WORDS=2, MEM_DATA=512-bit, WBUF_DIR_ENTRIES=4

- Achieved 11/11 test cases passing with zero UVM errors on the hpdcache_wrapper.sv DUT

- Next phase: extend the testbench to full CVA6 core-level verification with new test cases, requiring an ISA reference model and new UVM agents (estimated substantial effort)

- Updating the testplan to reflect the expanded core-level scope is upcoming

- A major debugging session resolved 17 distinct testbench bugs (compile failures, elaboration errors, runtime simulation issues) — all in the testbench, not the DUT

- Notable bugs fixed: interface type mismatches causing Questa ICE crashes; a hw_top hierarchy instantiation error; UVM analysis port/export type mismatch; an AXI R-channel deadlock from the memory model gating valid_i on ready_o; a write response FIFO missing that caused WBUF stalls under concurrent STOREs; scoreboard false-mismatches from X-propagation through uninitialized SRAM packed arrays

- TC 4.3 hash collision addresses derived from first principles using domino_pkg INDEX_WIDTH=10 XOR folding, yielding addr_P=0x1000, addr_Q=0x400000

- Known workaround: cfg_wbuf_threshold_i was manually edited in hpdcache_wrapper.sv, addressed via an intermediate signal fix

- QuestaSim Starter does not pass $argv through do commands, requiring separate .do files per test

THESIS PROJECT CACHE CONFIGURATION (Actual)

I-Cache (From CVA6 - cv32a6_imac_sv32_config_pkg.sv)

- IcacheLineWidth: 128 bits (16 bytes per cache line)

- IcacheByteSize: 16384 bytes (16 KB total I-Cache size)

- IcacheSetAssoc: 4-way associative

- Index width: $clog2(16384 / 4 / 16) = 8 bits (256 sets)

D-Cache / HPDcache (From hpdcache_config.svh in thesis repo)

- Cache Line Width: 512 bits = 64-bit word × 8 words per line

  - Source: CONF_HPDCACHE_WORD_WIDTH=64 + CONF_HPDCACHE_CL_WORDS=8

- Memory Bus Width: 512 bits (CONF_HPDCACHE_MEM_DATA_WIDTH)

- Cache geometry: 64 sets × 8-way associative

- Physical Address: 56 bits

- D-Cache size: 32 KB (implied from CVA6 config)

- Type: Write-Through with Domino Prefetcher integration

Key Architecture Insight

- I-Cache uses standard 128-bit line (narrow, cached instructions only)

- D-Cache uses 512-bit line (wide, supports high-bandwidth prefetching + domino memory pattern detection)

- This asymmetry enables aggressive prefetching on load data without stalling I-fetch